What’s Blocking and Why Do It?

With the effort you put into knitting, you want your finished item to look fabulous.  That’s where blocking comes in.  Blocking is an essential skill for knitters and crocheters.

In knitting, crochet and textile crafts, blocking is the process of wetting/soaking or steaming your finished garments. This sets their shape and stitches by being stretched out, and lay flat on a surface to dry. The blocking process evens the stitches and increases the level of drape. It can be used on lacework, sweaters, hats, scarves, mittens, and socks.

Blocking is the final step of making your project.  Blocking smooths and evens your stitches, sets the final dimensions, and gives your project that professional finished look. Don’t be intimidated by blocking. It is not a complicated process. The results are well worth the effort. You will be amazed what blocking does to those lace and cable designs.

Always block your finished pieces before seaming. By flattening and setting the shape of your pieces, you will be able to more easily line up your stitches to seam them together.

What blocking can’t do:

Blocking can smooth out stitches, but it won’t magically fix uneven tension

You can’t make a sweater 6 inches bigger than it was originally knit

Stockinette stitch worked without any borders is always going to curl. Always. No matter how much you block

Getting Started:

The type of fiber and the stitch pattern used factor into the best method for blocking your work. Often the pattern instructions will suggest a method. However, if you used a different yarn or the directions simply say “block,” you need to determine which blocking method to use.

Some fibers change their character significantly when blocked.  An example is alpaca yarn grows tremendously after blocking.  Many natural fibers bloom when blocked. When knitted the stitches in some fibers appear smaller and the fabric is more open and looser.  Once blocked, these same stitches will fill in the gaps and create a more solid fabric.  Use the pattern schematic and measurements to block to the size you want.

Keep in mind that some fibers block better than others. If you have difficulties opening your lace patterns, it could be the fiber used. While all fibers can be blocked and stretched, not all fibers stay in their ‘blocked’ shape. Natural fibers are some of the easiest fibers to block, and they will generally retain the shape and size they have been blocked to until they are re-blocked or they get wet.

Start by reading the yarn label.  Care instructions are usually written out or indicated by symbols. If the label gives instructions for machine washing and drying, follow these simple directions for easy blocking. If the label indicates handwashing and no machine drying, any of the methods listed here could be utilized.

Before you block your finished item experiment with blocking your gauge swatch before you block an actual knitted piece.

When blocking do not rub, twist, or wring a handknit. Doing so may distort the stitches beyond correction.

It is preferable to block individual pieces before sewing them together. Blocking makes the sewing process easier and the results of blocking are more consistent when you work with a single layer of fabric. You can block a garment that has been sewed together, but the results may not be as good.

Many experts warn against blocking ribbing, which will lose its natural elasticity if blocked while stretched open. However, ribbing can be successfully blocked if you squeeze it into its most contracted state (so that all the purl stitches recede behind the knit stitches) before you apply moisture.

Steam-block cables wrong side up. This may seem obvious, but just in case; if you are using a steamer or a steam-iron to block your cable knitting, do it with the wrong side of the cables facing upwards, or you will flatten the cables. Don’t press down; keep the iron or the steamer just a little bit above the fabric.

Allow the blocked handknit to air-dry completely before moving it.

One area that has different thoughts is the loose yarn ends.  Weaving those ends in before blocking helps to secure the ends in place.  However, blocking involves stretching the stitches.  If the ends are not woven in there is yarn to move if needed.

Blocking Tools:

The first decision is to determine where you will block your knits and what surface to use. The surface should be large enough to allow the piece to be spread out, resistant to water, and located in an out-of-the-way place so that the piece can be undisturbed until it is completely dry. If there are pets in the house, the best place for blocking may be behind a closed door.  You can purchase interlocking blocking mats that are sold in most craft stores and are very convenient to use.  Also available at hardware stores are floor mats in bigger sizes.  If you have a sewing cutting table, a great place to block.  You can also use a large garbage bag or plastic tablecloth over a guest bed or on the floor and placing a heavy bath towel on top of the plastic.

Rust proof T-pins are often recommended for use when blocking knitting. They are like regular straight pins except the head is shaped like a T. They are long and easy to work, so you don't have to worry about leaving them in your knitting while it dries.

For blocking lace and other large projects, you can purchase blocking wires, which are flexible metal wires that can help you block curves or the sides of a large project. The wires can be woven in and out of the project and reinforced with straight pins if necessary.

Blocking Techniques:

There are three main methods of blocking:  spray blocking, steam blocking, wet blocking.

## Spray Blocking AKA Misting Blocking

Spray blocking is the gentlest blocking process and is great for expensive and delicate fibers like silk and cashmere. It's a good method to use when you aren't sure what kind of yarn you're dealing with.

All you need to do is pin the piece to the desired dimensions and lightly spritz the finished piece with water from a spray bottle. Get it damp enough to relax the fibers, but not soaking wet. Allow to dry, and you're done.

## Steam Blocking

Steam blocking is a similar process to wet blocking; only you use steam to relax the fibers instead of water. This is the best process for fibers that shouldn't get wet, as well as for cottons, which tend to completely lose their shape when wet. It shouldn't be used on human-made fibers, because heat and steam tend to destroy them and make a mess of all your hard work.

Different knitters use different techniques when it comes to steam blocking. Some people stretch and pin their work to the desired shape before steaming, using the steam to help set the new shape. Others steam first and then pin, allowing the steam to relax the fibers and make it more pliable.

The method you use might depend largely on the flexibility of the knitting. If you can get it into shape without the steam, pin first. If not, steam and then pin.

The steaming method involves slightly dampening a clean sheet or another piece of fabric and placing it over the knitting. Use a hot iron to press very lightly on the sheet. Don't press like you are ironing; you're just pushing the steam through the sheet and into the knitting. Continue this process until the sheet is dry.

You also can steam block without a protective layer of fabric. Just set your iron on steam and wave the iron slowly over the knitting, being careful not to touch the work with the iron. Then pin, if necessary, and leave to dry.

## Wet Blocking AKA Immersion Blocking

Wet blocking knitting is an appropriate method for blocking man-made fibers, novelty yarns (except for some very delicate novelties that may fall apart when wet—check the label for care instructions and don't wet anything that says it shouldn't be wet) and some wools and wool blends if done carefully (remember heat and agitation can cause wool to felt).

Dampen the knitted piece so that it is wet but not dripping. Spread the piece out on a towel, sheet, or clean garbage bag (the bag won't absorb water, allowing the piece to dry faster) on the floor or a spare bed where it can sit undisturbed long enough to dry.

Gently stretch the piece as needed to meet your blocking goals. If you're trying to get a piece to the size or shape the pattern recommended, you might need a ruler, tape measure or yardstick to help you out.

Use rust-proof safety pins, straight pins or T-pins to hold the knitted piece to the towel or sheet. As the piece dries, it will retain the shape that you gave it.

Blocking non natural fibers

Blocking can only be applied to natural fibers. There’s a technique for synthetic fibers called ‘Killing’.  For reference “killing” acrylic means you melt the fibers so much they become limp and drape and no longer stretch (good for lace).

Acrylic yarn can be blocked, and depending on the project, should always be blocked. Doing this adds more drape to a knitted acrylic fabric.

The best method to block acrylic yarn is with heat, no matter if the piece is knit or crochet. The heat makes the fibers more susceptible to straightening out. Steam blocking or blocking with a hair dryer yield similar results.

I found that the best way to block acrylic yarn is with a hair dryer for both knit and crochet

- Lay out your fabric and pin into the desired shape.

- Plug in the hair dryer and turn it on to the high heat setting. Although, you could use a lower heat setting if you’re afraid of “killing” the acrylic. I used the high setting.

- Next, hover the hair dryer a few inches above the fabric.

- Move back and forth across the fabric, making sure to focus on the edges too. Do this for a couple of minutes.

- Press on the fabric to smooth out the edges.

- Turn off the hair dryer and let the fabric cool before removing the pins.

Blocking Acrylic and Others:

Acrylic and man-made fibers generally return to their pre-blocked shape quickly after the pins are removed and may not be the best choice for an intricate and heavily patterned lace shawl.

Here’s another method to consider.  Why spend extra time blocking when the washing machine and dryer will do it for you.  The item will eventually be cleaned using them.

Here’s how:

- Throw the fabric in the washing machine. (add other items that needed washing so to not waste water)

- Toss the fabric in the dryer right along with the other items.

- Remove the fabric when it’s still slightly damp.

- Place the fabric on a flat surface and press the fabric down flat.

- Let dry.

To conclude blocking is an essential step in the completion of a project which should never be ignored. It settles the fibers, evens out the fabric, give it more drape, and a polished, professional look. It can erase or lessen some problems in the fit and altogether reveal a pattern as we’ve seen with lace and cables. If you’ve never tried blocking before, please give it a go and I can promise that you will never look back.

Website Reference:

https://www.knitpicks.com/learning-center/blocking-basics